Graded Lab 1

Write an 8086 assembly language program that performs the following operations step by step:

- Store the following 16-bit numbers in memory starting at offset 1000H for current data segment:

- 0x1205, 0x0A3F, 0x00FF, 0x0B20

- Load these numbers one by one into registers and:

  - Add the first and second numbers,

  - Subtract the third number from the result,

  - Then add the fourth number.

- Store the final result in the register DX.

Code

.model small

.stack 100h

.data

num1 dw 0x1205

num2 dw 0x0A3F

num3 dw 0x00FF

num4 dw 0x0B20

.code

main proc

mov ax, @data

mov ds, ax

mov bx, 0x1000 ;offset

;store at memory

mov si, 0

mov ax, num1

mov [bx + si], ax

add si, 2

mov ax, num2

mov [bx + si], ax

add si, 2

mov ax, num3

mov [bx + si], ax

add si, 2

mov ax, num4

mov [bx + si], ax

;load first 2 numbers from memory

mov si, 0

mov ax, [bx + si]

add si, 2

mov dx, [bx + si]

add si, 2

;add first 2 numbers

add ax, dx

;load 3rd number from memory

mov dx, [bx + si]

add si, 2

;subtract 3rd number from result

sub ax, dx

;load 4th number from memory

mov dx, [bx + si]

;add 4th number to result and store in dx

add dx, ax

main endp

.exit

Storing at Memory

Output